Hybrid Coarse-to-Fine Profilometry for Rough Surfaces: A Coincidence-Proxy Height-Fidelity Benchmark on Measured Surfaces

Dawid Kucharski

Poznan University of Technology

dawid.kucharski@put.poznan.pl

Abstract
Optical profilometry remains an important route for surface-topography evaluation in precision manufacturing and functional-surface metrology [1]. Classical four-step phase-shifting interferometry (PSI) remains a strong practical baseline, and environmentally robust implementations are still an active issue in interferometric testing [2].  Its estimation accuracy is also limited by phase-step and photon-statistical constraints [3]. In parallel, quantum-entangled photon interferometry [4]  and high-visibility two-photon interference schemes [5]  have motivated nonclassical sensing architectures. What remains unclear is whether such channels improve practically relevant rough-surface metrology endpoints under fair comparison conditions.

This work presents a simulation benchmark of rough-surface profilometry comparing classical four-step phase-shifting interferometry (PSI), coincidence-proxy channels, and coarse-to-fine hybrid reconstruction under matched detected-count assumptions. The study uses 59 focus-variation (FV) topographies exported as Mountains/DigitalSurf.sur files as measured geometric references for simulated optical observations and is intended as a benchmark of reconstruction architectures rather than as a laboratory demonstration of quantum-enhanced metrology.

Figure 1 summarises the main robust result, which is limited to height fidelity: on the measured-surface benchmark, the hybrid branch gives the lowest surface-level median detrended height RMSE (314 nm), wins height RMSE on 32 of 59 surfaces, and remains below an optimistic classical-frontier oracle assembled from stronger auxiliary classical workflows (559 nm). Roughness-endpoint rankings are weaker and must be read cautiously. Figure 2 shows the corresponding native-grid roughness comparison. A matched-bandwidth control shows that the apparent native-grid Sq advantage of the direct coincidence-proxy branch is largely explained by reference-bandwidth mismatch. Treatment holdouts reduce agreement to 50.0% for ΔSa and 0.0% for ΔSq, and even the best native-grid ΔSz branch exceeds the descriptor-scaled tolerance band on 74.6% of surfaces. Classical two-colour and classical-frontier controls further show that envelope-following behaviour is not uniquely nonclassical.

A rate-based coincidence control yields the same conclusion: under ideal matched-count rates, the hybrid branch remains best in height RMSE (291 nm), while under non-ideal detector assumptions, the direct branch degrades to 1308 nm and the hybrid to 376 nm. The supported design rule is therefore architectural and narrow: within the present proxy benchmark, coincidence-derived information is most defensible as a coarse absolute-height prior inside a hybrid estimator when fixed-workflow height fidelity is the primary endpoint. Experimental validation remains necessary before stronger quantum-metrology claims can be made.

Keywords: coincidence-proxy interferometry; simulation benchmark; surface profilometry; hybrid reconstruction; coarse-to-fine reconstruction; phase-shifting interferometry; synthetic wavelength; roughness metrology; focus-variation microscopy; areal surface texture

Figure 1. Measured-surface benchmark summary across all FV *.sur samples: height RMSE distributions by method and RMSE trends vs surface roughness. All values are computed from the measured height maps (after plane detrending).

Figure 2. Direct comparison of areal roughness parameters estimated from the simulated interferometric reconstructions against the corresponding values computed from the Alicona reference height maps. Each point is one measured *.sur surface; the dashed line marks perfect agreement. This native-grid view is diagnostic rather than definitive for workflow selection because the interferometric forward model operates on the downsampled benchmark grid. The hybrid branch remains strongest for Sa, whereas the direct coincidence-proxy branch more often follows the native-grid Sq and Sz values because it retains broad-envelope excursions more readily than fine texture.

Funding: This work was supported by a grant from the Polish Ministry of Science under the programme “Polish Metrology II” (Polska Metrologia II,

Poland), project no. PM-II/SP/0090/2024/02.

References

[1] Vorburger T V and Teague E C 1981 Optical techniques for on-line measurement of surface topography. Precision Engineering 3 61–83

[2] McDonnell E M and Deck L L 2020 Solutions for environmentally robust interferometric optical testing Proceedings of SPIE - The International Society for Optical Engineering 11487

[3] Okamoto R and Tahara T 2021 Precision limit for simultaneous phase and transmittance estimation with phase-shifting interferometry Physical Review A 104

[4] Richards R K 2004 Quantum-entangled photon interferometry Interf. XII: Tech. Anal. 5531 17–23

[5] Rarity J G, Burnett J, Tapster P R and Paschotta R 1993 High-visibility two-photon interference in a single-mode-fibre interferometer EPL 22 95–100